| S203A S203AM LARGE FULL SIZE FLAT SWITCH LOCK. |
| --- |
| DESCRIPTIONS: - Zinc alloy die cast housing and cylinder. - Bright chrome plated standard. - 11 disc tumbler mechanism. - Reversible double-bitted brass keys - Key may be withdrawn in one or both positions. - S203A 2 position (off-on) 2 spade terminals. - S203AM 2 position (off-on) 2 spade terminals, spring loaded for momentary contact. - Hex nut mounted. - 5 combinations available.  SPECIFICATION - Contact Rating: 4A 125VAC, 2A 250VAC. - Contact Resistance: 10Milli-Ohms Max. - Insulation Resistance: 500 Meg-Ohms Min. (500VDC) - Dielectric Strength: 1500 VAC -1 Minute.D KEY-PULL FIGURES -Off Position -Indicates Key-Pull -Indicates No Key-Pull |  |

| Fig. No. | Lock Number | Pole | Positions | Terminals | Key Pulls | Turn |  |  | Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S203A-1 | 1 | 2 | 2 | 1 | 90  | Off | On | SPST |
| 2 | S203A-2 | 1 | 2 | 2 | 2 | 90  | Off | On | SPST |
| 3 | S203AM | 1 | 2 | 2 | 1 | 90  | Off | (On) | SPST |